WAP Database Watershed Statistic Documentation

All GIS base datasets and ArcToolbox ModelBuilder tools archived in GIS library - #1567

Spreadsheets with statistical results for each watershed saved in FileSite (see FS # after each characteristic).

Slope

Average Slope (degrees) – 10 meter National Elevation Dataset (NED)

FS # 351564

Base Flow Index (BFI)

Percent – USGS

FS # 351564

Elevation

Mean elevation (feet) – 10m NED

FS # 351564

PE

Potential Evapotranspiration (inches) – Global-PET, A. Trabucco

FS # 351564

Climatic

Annual Mean Maximum Temperature (Fahrenheit)

Annual Mean Minimum Temperature (Fahrenheit)

Annual Mean Precipitation (inches)

Monthly Mean Precipitation (inches)

PRISM Climate Group Oregon State

30-Year Normals (1980-2010)

FS # 351578

STATSGO Soils

Available Water Capacity (inches/inch)

Clay in Soil (percent)

Soil Erodibility Factor (unitless)

Soil Permeability (inches/hour)

Soil Thickness (inches)

Hydrologic Soil Group (unitless)

from ussoils, Schwarz and Alexander, 1995 (includes these calculated averages)

To understand the procedures used to create the attribute table USSOILS.MUID_ATTS, it helps to have an understanding of the STATSGO data structure.  The smallest spatial entity mapped within the STATSGO dataset is called a map unit, of which there are 10,498 broken out into 78,518 polygons within the conterminous United States.  Each map unit consists of up to 21 components.  Components are used to apportion different characteristics of a map unit and do not represent a separate spatial entity.  To help understand this, consider a map unit that has only two components.  Suppose 30 percent of this map unit contains soil of type A and the remaining 70 percent contains soil of type B.  Thus, component 1 of the map unit would have soil type A and component 2 would have soil type B.  The data base would record the soil type for each component, along with the percentage of the map unit represented by each component (in this example, 30 percent for component 1 and 70 percent for component 2).  The specific locations of soil types A and B are not spatially defined in the data base.  Each component is further delineated into a maximum of 6 layers, corresponding to distinct soil layers.  Soil characteristics recorded for a layer typically consist of a high and low value, which describes a range for that characteristic within that layer.  Included among the characteristics of the soil layer is the thickness of the layer.  In developing the USSOILS.MUID_ATTS table, it is necessary to aggregate the layer and component information up to the level of a map unit.  To aggregate layer information to the component level, we first construct a simple average of the high and low values of a characteristic for each layer.  We then construct a weighted average across all layers of the layer-specific characteristic averages, the weights being the thickness of the layer relative to the total thickness of all layers.  For component-level information that is expressed as a range, we construct a simple average of the high and low values defining the range. These steps reduce layer-specific information and ranges of component-specific information into a single datum of component-specific information for each characteristic in the data base.  Finally, to obtain map unit information, we construct a weighted average across all components of the component-specific information, the weights being the component percentages given for the map unit.

Hydrologic Soils - A code identifying the hydrologic characteristics of the soil.  The character codes defined in the STATSGO component file are converted into numeric codes according to Bill Battaglin's methods (Battaglin, written communication dated March 8, 1995).  The coding transformations are A = 1 (high infiltration, deep soils, well drained to excessively drained sands and gravels), B = 2 (moderate infiltration rates, deep and moderately deep, moderately well and well drained soils with moderately coarse textures), C = 3 (slow infiltration rates, soils with layers impeding downward movement of water, or soils with moderately fine or fine textures), D = 4 (very slow infiltration rates, soils are clayey, have a high water table, or are shallow to an impervious layer),  Mixture codes A/D, B/D, and C/D are assigned the value 4.  Also, we subselect certain areas and assign values for hygrp based on the area type.  Miscellaneous areas (denoted by M in compkind) and labeled as Dumps, and Gullied Land in compname are assigned the hygrp = 2.5 if the hydgrp value is missing.  Areas denoted as Pits, Rock Outcrops, Terrace Escarpments, and Urban land with missing hydgrp are assigned a hygrp of 4. See the documentation of the SAS program "setussoils.sas" for additional details.  The transformed data are averaged across components using the component percentage as weights. The missing value indicator is - 0.1.

FS # 351565

Glacial

Percent – PADCNR Map 59, 1997

http://www.dcnr.state.pa.us/topogeo/index.aspx

FS # 351572

Alluvium

Percent – PADCNR hardcopy maps 1975-1993

http://www.dcnr.state.pa.us/topogeo/index.aspx

NYS Museum – surficial geology 1999

material codes – al, fg, ls, og, pm

https://www.nysm.nysed.gov/gis/

FS # 351572

Till

Percent – PADCNR, NYS Museum

Created using glacial and alluvium datasets, till is glacial area that is not alluvium

FS # 351572

Stream Density

mi/mi2 – USGS NHD high resolution

FS # 351579

Drainage Area

mi2 – USGS 10m NED

FS # 351579

Stream Miles

Miles – USGS NHD high resolution

FS # 351579

Stream Order

At watershed outlet – SRBC

FS # 351579

Aquatic Resource Class (ARC)

At watershed outlet – SRBC

FS # 351579

Physiographic Divisions

Percent – Fenneman Physical Divisions of the United States

FS # 351577

Geology

Percent - bedrock geology SRBC

based on NYS Museum, NYS Geologic Survey 1999; PA DCNR geologic formations 2001; USGS 1992

Rock Type Categories:

Conglomerate

Dolomite

Limestone

Quartzite

Sandstone

Shale

Other

FS # 351571

Coal

Percent – PADEP

FS # 351566

Ecoregion III and IV

Percent – USEPA, Omernik

FS # 351569, FS # 351570

Designated Streams

Miles – PADEP Chapter 93 streams 2013; NYSDEC stream classifications 2015, MDE designated streams 2012

FS # 351568, FS # 351567

Impaired Streams

Miles – PADEP Integrated List 2016, NYSDEC Priority Waterbody List 2014, MDE Impaired Streams 2012

FS # 351574

Land Use

Percent - Chesapeake Bay Watershed Land Cover Data Series. 2006. Created by MDA Federal, Inc. and funded by USGS. Reston, VA.

FS # 351576